| Huawei eSight for vROps Plug-in |  |  |
| --- | --- | --- |
| User Guide |  |  |
| V2.0.1 |  |  |
| Issue:01 Date:2021-01-27 |  |  |
|  | Copyright © Copyright Holder. All rights reserved. |  |

|  | Notice |  |  |
| --- | --- | --- | --- |
|  | The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute the warranty of any kind, express or implied. |  |

Contents

Preface	1

1 Overview	1-1

2 Installing and Uninstalling the Huawei vROps Plug-in	2-1

2.1 Installing the Huawei vROps Plug-in	2-1

2.2 Uninstalling the Huawei vROps Plug-in	2-4

3 Configuring an eSight Instance	3-1

3.1 Addition	3-1

3.2 Modification	3-5

3.3 Deletion	3-7

4 Querying Server Information	4-1

4.1 Querying the Server List	4-1

4.2 Querying Component Information	4-3

4.3 Adding a Component View	4-6

4.4 Locating Alert Information	4-11

5 Querying the Huawei vROps Plug-in Version	5-1

6 FAQs	6-1

6.1 Connection Test Failed When Adding an eSight	6-1

A Getting Help	1

A.1 Collecting Fault Information	1

A.2 Preparing for Debugging	1

A.3 Using Product Documentation	1

A.4 Obtaining Technical Support	1

Preface

Overview

This guide describes how to install and maintain Huawei vRealize Operations Manager (vROps) plug-in.

Intended Audience

This document is intended for:

Technical support engineers

System maintenance engineers

Symbol Conventions

The symbols that may be found in this document are defined as follows.

| Symbol | Description |
| --- | --- |
|  | Indicates an imminently hazardous situation which, if not avoided, will result in death or serious injury. |
|  | Indicates a potentially hazardous situation which, if not avoided, could result in death or serious injury. |
|  | Indicates a potentially hazardous situation which, if not avoided, may result in minor or moderate injury. |
|  | Indicates a potentially hazardous situation which, if not avoided, could result in equipment damage, data loss, performance deterioration, or unanticipated results. NOTICE is used to address practices not related to personal injury. |
|  | Calls attention to important information, best practices, and tips. NOTE is used to address information not related to personal injury, equipment damage, and environment deterioration. |

Change History

| Issue | Date | Description |
| --- | --- | --- |
| 01 | 2021-01-27 | This issue is the first official release. |

# Overview

Function Description

The Huawei eSight for vROps plug-in is a plug-in integrated in the vRealize Operations Manager software and used for Huawei server management. By adding eSight, it can monitor Huawei servers. Only vRealize Operations Manager 8.0 and 8.1 are supported. In this document, vRealize Operations Manager 8.1 is used as an example.

You can implement the following functions by using the Huawei eSight for vROps plug-in:

Query server lists.

Query basic information of server components.

Query alarm information and health status of servers.

Supported Servers

Table 1-1 lists the servers supported by the Huawei eSight for vROps plug-in.

Supported servers

| Type | Server Model |
| --- | --- |
| Rack server | RH1288 V3 |
|  | RH2288H V2 |
|  | RH2288 V3 |
|  | RH2288H V3 |
|  | RH5885 V3 |
|  | RH5885H V3 |
|  | RH8100 V3 |
|  | 1288H V5 |
|  | 2288H V5 |
|  | 2488 V5 |
| Blade server | CH121 V3 |
|  | CH242 V3 |
| Chassis | E9000 (MM910) |
| High-density server | XH321 V3 |
|  | XH620 V3 |
|  | XH622 V3 |
|  | XH628 V3 |
| KunLun server | 9008 |
|  | 9016 |
|  | 9032 |

# Installing and Uninstalling the Huawei vROps Plug-in

2.1  Installing the Huawei vROps Plug-in

2.2  Uninstalling the Huawei vROps Plug-in

## Installing the Huawei vROps Plug-in

            - Access GitHub and download the latest Huawei vROps plug-in installation package, such as Huawei_eSight_For_vROps_X.X.X.zip.

Decompress the installation package.

Obtain the installer, such as HuaweieSightManagementPack_X.X.X.pak.

On the vROps WebUI, choose Administration.

The Administration page is displayed.

In the navigation pane on the left, choose Repository.

The Repository page is displayed.

In the lower part of the page, click ADD/UPGRADE.

The Add Solution dialog box is displayed, as shown in Figure 2-1.

Add Solution

On the Select Solution tab page, click BROWSE and select the plug-in installation file.

Select Install the PAK file even if it is already installed.

Click UPLOAD and upload the installation file.

Click NEXT.

The Confirmation dialog box is displayed.

Click Yes.

The End User License Agreement tab page is displayed, as shown in Figure 2-2.

End User License Agreement

Select I accept the terms of this agreement and click NEXT.

The Installation tab page is displayed.

After the installation is complete, click FINISH.

In the lower part of the Repository page, view the installed Huawei vROps plug-in, as shown in Figure 2-3.

Viewing the installed vROps plug-in

----End

## Uninstalling the Huawei vROps Plug-in

            - On the vROps WebUI, choose Administration.

The Administration page is displayed.

In the navigation pane on the left, choose Repository.

The Repository page is displayed.

In the lower part of the page, select the Huawei vROps plug-in to be uninstalled and click UNINSTALL, as shown in Figure 2-4.

Uninstalling the vROps plug-in

The Warning dialog box is displayed, as shown in Figure 2-5.

Warning dialog box

Select I understand the risk and agree. and click OK.

The Huawei vROps plug-in is uninstalled.

----End

# Configuring an eSight Instance

3.1  Addition

3.2  Modification

3.3  Deletion

## Addition

            - Set the whitelist.

By default, a whitelist of eSight northbound interfaces is configured. To add an eSight system properly, you must add the IP address of the server where vROps is located to the whitelist of eSight northbound interfaces.

- Log in to the eSight WebUI.

- Choose System > Northbound Intergration > Third-party System > Create.

The Third-party System page is displayed, as shown in Figure 3-1.

Third-party System

- Set the following parameters:

IP address: Set it to the IP address of the vROps server.

Protocol type: Select HTTPS and deselect other protocols.

System ID: Retain the default value or enter a new value. The value can be an IP address or a string of 1 to 64 characters, including digits 0-9, lowercase letters a-z, uppercase letters A-Z, and special characters @_- (), .^$~`!.

- Click OK.

The IP address of the vROps server is successfully added to the whitelist, as shown in Figure 3-2.

Setting successful

On the vROps WebUI, choose Administration.

The Administration page is displayed.

In the navigation tree on the left, choose Other Accounts.

The Other Accounts page is displayed.

Click ADD ACCOUNT.

Select the type of the account to be added, for example, eSight Adapter.

The New Account page is displayed, as shown in Figure 3-3.

New Account

Set the following parameters:

Name: user-defined name to identify different eSight instances

Description: (optional) user-defined description to describe an eSight instance

Server IP: eSight IP address

Server Port: eSight port number, which is 32102 by default.

In the Credential field, click .

The Manage Credentials dialog box is displayed, as shown in Figure 3-4.

Manage Credentials

Set the following parameters and click OK.

Credential name: user-defined name to identify different credentials

Username: username of the eSight open API to be added. The default username is openApiUser.

Password: password of the eSight OpenAPI to be added. The default password is Changeme_123.

Click VALIDATE CONNECTION.

If the connection is successful, a test connection success dialog box is displayed. Click OK.

If the connection fails, check whether the eSight IP address, port number, username, and password are correct.

If no, modify them and click VALIDATE CONNECTION again.

If yes, contact technical support.

Click ADD.

The eSight instance is added.

On the Other Accounts page, view the added eSight instance, as shown in Figure 3-5.

Viewing the added account

----End

## Modification

            - On the vROps WebUI, choose Administration.

The Administration page is displayed.

In the navigation tree on the left, choose Other Accounts.

The Other Accounts page is displayed, as shown in Figure 3-6.

Other Accounts

Click the name of the eSight instance to be modified, for example, esight221.5.

The eSight instance information page is displayed, as shown in Figure 3-7.

eSight instance information page

Modify the following parameters as required:

Name: instance name

Description: instance description

Server IP: eSight IP address

Server Port: eSight port number

Credential: Click  to change the credential name, eSight username, and password.

If the eSight port number, username, or password is changed after the eSight connection, you need to synchronize the new port number, user name, or password to the eSight. Otherwise, server management is affected.

Check whether the information in the Connect Information area is modified.

If yes, click VALIDATE CONNECTION.

If the connection is successful, a test connection success dialog box is displayed. Click OK.

If the connection fails, check whether the eSight IP address, port number, username, and password are correct.

If no, modify them and click VALIDATE CONNECTION again.

If yes, contact technical support.

If no, go to Step 6.

Click SAVE.

The Other Accounts page is displayed. You can view the modified eSight information.

----End

## Deletion

            - On the vROps WebUI, choose Administration.

The Administration page is displayed.

In the navigation tree on the left, choose Other Accounts.

The Other Accounts page is displayed, as shown in Figure 3-8.

Other Accounts

Click  next to the name of the eSight instance to be deleted.

The operation selection list is displayed.

Click Delete.

A confirmation dialog box is displayed, as shown in Figure 3-9.

Deletion confirmation dialog box

Select Delete related objects. (which is selected by default) and click DELETE.

The eSight instance is deleted.

----End

# Querying Server Information

4.1  Querying the Server List

4.2  Querying Component Information

4.3  Adding a Component View

4.4  Locating Alert Information

## Querying the Server List

            - On the vRealize Operations Manager WebUI, select Environment.

The Environment page is displayed.

In the navigation tree, choose Huawei eSight Server Management > Device Inventory tree.

The Device Inventory tree page is displayed.

In the navigation tree, query the server list, as shown in Figure 4-1.

Device Inventory tree

1: eSight IP address

2: Server type. blade indicates a blade server, highdensity indicates a high-density server, and rack indicates a rack server.

3: Server model and iBMC IP address

4: Component group

5: A component

6: IP address and slot number of a management module

7: Device group

8: A device

----End

## Querying Component Information

            - On the vRealize Operations Manager WebUI, select Environment.

The Environment page is displayed.

In the navigation tree, choose Huawei eSight Server Management > Device Inventory tree.

The Device Inventory tree page is displayed.

Choose eSight IP address > Server type > Server IP address > Component group > Component to be viewed, as shown in Figure 4-2.

Component information

View the summary of the component.

- Click Summary.

The Summary page is displayed.

- In the Recommended Actions area, view the running status and alarm information about the component, as shown in Figure 4-3.

Viewing the summary

View warning information of the component.

- Click Alerts.

The Alerts page is displayed.

- Select a grouping mode, for example, Object Type.

- Click  to expand the warning list.

- Click a warning name to view details about the warning.

Viewing warning information

View the real-time status curve of the component.

- Click Metrics.

The Metrics page is displayed.

Metrics

- Click  to expand the information list under All Metrics or All Properties.

- Double-click the real-time status information to be viewed. For example, double-click Health, as shown in Figure 4-6.

You can view the real-time status curve in the right pane.

Viewing the real-time status curve

View details about a component view.

- Click more > Details.

The Details page is displayed.

- Select a view.

- View details about the view.

Viewing details about the view

----End

## Adding a Component View

            - On the vROps WebUI, choose Environment.

The Environment page is displayed.

In the navigation tree on the left, choose Huawei eSight Management Pack > Device Inventory tree.

The Device Inventory tree page is displayed.

Select an eSight IP address, a server model, a server IP address, a component group, and a component, as shown in Figure 4-8.

Device Inventory tree

In the right pane, choose more... > Details.

The Details page is displayed, as shown in Figure 4-9.

Details

Click ADD.

The New View dialog box is displayed, as shown in Figure 4-10.

New View

On the Name and Description tab page, enter the name and description of the new component view.

On the Presentation tab page, select List and view the preview effect on the right, as shown in Figure 4-11.

Presentation

On the Subjects tab page, click eSight Adapter and choose the type of the component to be added.

For example, to add a component view to PSU1, choose eSight Adapter > Huawei PSU.

On the Data tab page, select Metrics or Properties, double-click the property marked with , and view the new property in the Data area, as shown in Figure 4-12.

Data

To delete a property, click  in the corresponding property column in the Data area. For example, to delete the added Health Status, click  in the Health Status column.

On the Visibility tab page, retain the default settings and click SAVE.

On the Details page, view the added component view.

In Device Inventory tree, select a server IP address. In the right pane, choose Details and click the added component view to check all views of this component type of this server, as shown in Figure 4-13.

Viewing the component view 1

In Device Inventory tree, select a server type. In the right pane, choose Details and click the added component view to check all views of this component type of this server type, as shown in Figure 4-14.

Viewing the component view 2

In Device Inventory tree, select an eSight IP address. In the right pane, choose Details and click the added component view to check all views of this component type of this eSight instance, as shown in Figure 4-15.

Viewing the component view 3

----End

## Locating Alert Information

            - On the vROps WebUI, choose Alerts.

The Alerts page is displayed.

In the navigation pane, choose Triggered Alerts.

The Triggered Alerts page is displayed.

Select Object Type from the Group By drop-down list box.

In the alarm column of the component for which the alarm is generated, click  to expand the list and click Triggered On, as shown in Figure 4-16.

You can only select a component, not a component group. Otherwise, the alerted server cannot be located.

All alerts

The page about the triggered object is displayed, as shown in Figure 4-17.

Triggered object

In the navigation tree on the left, click an object (such as Huawei Server Group or Huawei Server) to view the server to which the trigger object belongs.

----End

# Querying the Huawei vROps Plug-in Version

            - On the vROps WebUI, choose Administration.

The Administration page is displayed.

In the navigation pane on the left, choose Repository.

The Repository page is displayed.

Check the current version of the Huawei vROps plug-in, as shown in Figure 5-1.

Viewing the plug-in version

----End

# FAQs

6.1  Connection Test Failed When Adding an eSight

## Connection Test Failed When Adding an eSight

Symptom

After an eSight server is added, a failure message is displayed during the connection test.

Cause

The user name or password is incorrect.

The eSight northbound interface user is locked.

A whitelist has not been set.

Solution

The user name or password is incorrect.

Enter the user name and password of the eSight northbound interface user. To view the user name and password, perform the following steps:

  - Log in to the eSight WebUI.

  - Choose System > User Management > User. The User page is displayed.

The role of the eSight northbound interface user is Open API user group, and the user name is displayed under User Name.

Viewing information about the eSight northbound interface user

  - Click  to display the dialog box for resetting the user password.

Reset Password

  - Enter a password in New password and Confirm password.

  - Click OK. The password is reset.

The eSight northbound interface user is locked.

To unlock a user, perform the following steps:

  - Log in to the eSight WebUI.

  - Choose System > User Management > User.

The User page is displayed.

  - Click  to set Status of the eSight northbound interface user to Enabled.

Unlocking a user

A whitelist has not been set.

By default, a whitelist of eSight northbound ports is configured. To add an eSight system properly, you must add the IP address of the server where vROps is located to the whitelist of eSight northbound ports.

  - Log in to the eSight WebUI.

  - Choose System > Northbound Integration > Third-party System > Create.

The Third-party System page is displayed, as shown in Figure 6-4.

Third-party System

  - Set the following parameters:

IP address: Set this parameter to the IP address of the vROps server.

Protocol type: Select HTTPS and deselect other protocols.

System ID: Retain the default value or enter a new value. The value can be an IP address or a string of 1 to 64 characters, including digits (0-9), lowercase letters (a-z), uppercase letters (A-Z), and special characters @_- (), .^$~`!.

  - Click OK.

The IP address of the vROps server is set as a whitelist, as shown in Figure 6-5.

Set successfully

Getting Help

If you encounter any problems during routine maintenance or troubleshooting, contact Huawei technical support engineers.

  - Collecting Fault Information

Before troubleshooting, obtain the following information:

Customer company and address

Contact person and telephone number

Time when the fault occurred

Detailed fault symptom

Device type and software version

Any measures taken and effects

Fault severity and expected rectification deadline

Preparing for Debugging

When you seek Huawei technical support, Huawei technical support engineers may assist you in performing some operations to further collect fault information or rectify the fault.

Before contacting technical support engineers, prepare the spare parts for boards and port modules, screwdrivers, screws, serial cables, and network cables.

Using Product Documentation

Huawei provides the documents delivered with the equipment. This document provides guidance for you to solve common problems that occur during routine maintenance or troubleshooting.

To better rectify the fault, you are advised to use the guide before contacting Huawei technical support engineers.

Obtaining Technical Support

Huawei's timely and efficient response is available from:

Local branch offices

Secondary technical support system

Telephone support

Remote support

Onsite support

Technical Support Website

Obtain technical documents at Huawei Technical Support website.

Self-Service Platform and Community

Learn more about servers and communicate with experts at:

Computing Product Information Service Platform for specific server product documentation.

Huawei Enterprise iKnow for quick learning about products.

Huawei Enterprise Support Community (Servers) for learning and discussion.

News

For notices about product life cycles, warnings, and updates, visit Support > Bulletins > Product Bulletins.

Cases

Learn about server applications at Computing Case Library.

The Computing Case Library is available only to Huawei partners and Huawei engineers.

Huawei Technical Support

If a fault persists after taking the above measures, obtain technical support in the following ways:

Contact Huawei customer service center.

Enterprise customers

Send emails to support_e@huawei.com or visit Global Service Hotline.

Carriers

Send emails to support@huawei.com or visit Global TAC Information.

Contact technical support at your local Huawei office.